The Build Company — Website Pre-Launch QA & Packages Verification

Executive Verification Dossier | thebuildcompany.in | Chennai, Tamil Nadu

# 1. 3-Tier Construction Packages Technical & Commercial Verification

Detailed technical specification comparison, material allowance validation, feasibility in Chennai, and legal risk audit across all 12 construction categories:

| Cat # | Category | Basic Plan (₹1,899/sqft) | Standard Plan (₹2,199/sqft) | Premium Plan (₹2,599/sqft) | Chennai Feasibility & Cost Audit | Legal & Compliance Risk | Client Confirmation Question |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | Structure & RCC | Fe 550 Steel, Coromandel/Dalmia 53G, M20 Concrete | Fe 550D Seismic Resilient Steel, Grade 53 Ultratech/Coromandel, M25 Mix with Integral Waterproofing | Fe 550D Corrosion Resistant Steel, Ultratech Super Grade 53, M25/M30 Mix with Crystalline Admixtures | Feasible. Fe 550D in coastal Chennai is vital for corrosion resistance. M25 mix conforms to IS 456. | High. Substituting Fe 550D with secondary steel on site represents severe structural liability. | Do you provide independent NABL lab 7-day and 28-day concrete cube compressive strength test certificates to clients? |
| 02 | Flooring | 2x2 ft Double Charged Vitrified Tiles (₹55/sqft) | 4x2 ft Glazed Vitrified Tiles (₹90/sqft) / Wooden Laminate in Master Bedroom | Italian Marble / Large Porcelain Slabs (₹220/sqft) + Solid Hardwood | Feasible. Material allowances (₹55, ₹90, ₹220/sqft) are realistic in Chennai tile markets. | Medium. Clients often select costlier tiles. Rate must clarify if laying & 10% wastage are included. | Are laying, adhesive, and skirting charges included when homeowners upgrade tiles above supply allowance? |
| 03 | Main & Internal Doors | Standard Teak Wood Main Frame & Shutter; Flush Internal Doors | First-Quality Teak Main Door with Brass Hardware; Molded Veneer Internal Doors | Handcrafted 8-Foot Teak Main Door with Biometric Smart Lock; Full Flush Teak Veneer | Teak prices vary widely. Must specify teak species (Burma, African, or Country Teak). | High. Stating 'First Quality Teak' without species creates disputes over knot defects. | What specific teakwood grade (Ghana/African Teak or Country Teak) is standard in the ₹2,199 package? |
| 04 | Windows & Glazing | Standard 2-Track UPVC Sliding Windows with Bug Mesh | Heavy 3-Track UPVC / Thermal Break Aluminum with Toughened Glass | Acoustic Double-Glazed Argon-Filled Slimline European Aluminum Systems | Feasible. In dusty Chennai corridors, 3-track with SS bug mesh is essential. | Low. UPVC profiles should specify 10-year UV degradation warranty. | What brand of UPVC profiles (Fenesta, Prominance, Kommerling) is officially supplied? |
| 05 | Electrical & Smart | Finolex / Anchor Concealed Wiring; Anchor Roma Switches | Havells / Polycab FRLS Wiring; Legrand Arteor / Schneider Switches | Complete Smart Automation Conduit Prep, Legrand / Lutron Switches, EV Charging Provision | Feasible. FRLS wiring in Standard tier is standard urban practice. | Medium. Excludes TNEB 3-phase utility meter deposits (₹40,000–₹80,000). | Does turnkey electrical scope cover internal conduit/wiring only, excluding TNEB utility meter charges? |
| 06 | Plumbing & Drainage | Ashirvad / Supreme CPVC Pipes, 1,000L Overhead Tank | Ashirvad FlowGuard CPVC Pipes, Dual Water Supply, 2,000L Tank | Pressurized Water System, Soundproof Waste Pipes, Automated Level Sensors | Feasible. Dual line (groundwater/potable water) is critical due to Chennai water TDS. | High. Underground sump volume is not stated (10,000L RCC vs 4,000L brick creates large cost gap). | What standard capacity and construction type (RCC vs Brick) is included for the underground water sump? |
| 07 | Kitchen | Polished Black Granite Counter (20mm), Nirali SS Sink, 2ft Dado | Premium Quartz / Granite Counter, Double Bowl SS Sink, 4ft Dado | Designer Quartz Counter, Island Provision, Franke/Grohe Granite Sink | Feasible. Modular kitchen woodwork (cabinets, chimney) is excluded. | Medium. Homeowners frequently confuse 'turnkey kitchen' with modular cabinets. | Confirm that modular kitchen cabinets, chimney, and hob are explicitly excluded from base rate. |
| 08 | Bathrooms & Sanitary | Parryware / Hindware EWC, Standard Wall Mixer | Jaquar / Kohler Wall-Hung EWC with Concealed Cistern, Single Lever Diverter | Grohe / Kohler Thermostatic Diverters, Toughened Glass Partitions | Feasible. Kohler/Jaquar wall-hung concealed cisterns offer high conversion appeal. | Medium. Toughened glass shower enclosures must specify thickness (8mm vs 10mm). | Are vanity counters, mirrors, and exhaust fans included, or sanitaryware supply only? |
| 09 | Painting & Finishes | 2 Coats Asian Paints Tractor Emulsion, Ace Exterior | 2 Coats Asian Paints Royale Luxury Emulsion, Apex WeatherProof | Asian Paints Royale Aspira with Textures, Apex Ultima Protek (10-Yr Guarantee) | Feasible. Royale Emulsion (Standard) and Tractor (Basic) are standard contractor benchmarks. | Low. Wall putty (2 coats Birla White) and primer must be confirmed before paint coats. | Does painting scope include 2 coats of wall putty plus 1 coat primer before emulsion coats? |
| 10 | Architecture & Elevation | 2D Floor Plans, Standard 3D Elevation | Bespoke 3D Elevation with Louver / CNC Detailing, Structural Drawings, Vastu | Full Package: 3D Walkthrough Video, Landscape Layout, Lighting & Ceiling Plans | Feasible. Premium tier includes 3D video walkthrough and false ceiling drawings. | High. CMDA plan sanction fees are excluded; municipal liaison assistance included. | Does architectural package include structural stability certificates signed by licensed engineer for bank loans? |
| 11 | Project Supervision | Weekly Site Civil Engineer Audits, Milestone Verification | Full-Time Dedicated Site Supervisor, Weekly WhatsApp Video Reports, Cube Tests | Dedicated Senior Project Architect, Daily Digital Logbook, 3rd-Party Audit | Crucial differentiation. Standard tier has full-time supervisor; Basic has weekly audits. | Critical. Clients choosing Basic (₹1,899) must know there is no full-time daily site supervisor. | In Basic package (₹1,899), how many weekly visits are contractually guaranteed by the civil engineer? |
| 12 | Warranty & Support | 5-Year Structural Warranty | 5-Year Comprehensive Structural & Waterproofing Warranty + 1-Year Free Maintenance | 5-Year Full Turnkey Warranty + 10-Year Structural Assurance + Lifetime Civil Advisory | Severe legal exposure. 'Lifetime Civil Advisory' and '10-Year Assurance' are hard to enforce. | Critical. Harmonize to single clear standard: 5-Year Written Structural Warranty + 1-Year Maintenance. | Does your firm execute an official written Structural Warranty Certificate at handover? |

# 2. Top 10 Critical Items to Fix Before Deployment

1. Replace Dummy Phone & WhatsApp Links: Update '+91 98400 12345' and 'wa.me/919840012345' across config/site.ts with client's real phone lines.

2. Connect Active Lead Webhook Destination: Set LEAD_WEBHOOK_URL in server .env to route incoming leads directly to CRM, Zapier, or Google Sheets.

3. Clarify GST (18%) Across All Pricing Displays: Explicitly append '+ 18% GST extra as applicable' beside all sq.ft rates (₹1,899, ₹2,199, ₹2,599).

4. Replace or Watermark Stock Project Images: Replace Unsplash stock photos with actual site photos, or watermark as '3D Architectural Elevation Concept'.

5. Verify Real Founder & Architect Credentials: Obtain real founder engineering degrees and Council of Architecture (CoA) license numbers.

6. Deploy Dedicated Privacy Policy & Terms Pages: Replace footer '#privacy' and '#terms' anchors with dedicated, legally sound /privacy-policy and /terms routes.

7. Add Statutory Registration Identifiers to Footer: Display GSTIN, TNRERA registration, MSME/CIN, and physical office address with pincode in footer.

8. Qualify '100% Price Lock / Spikes Absorbed' Guarantee: Add standard contract clause: 'Raw material price protection locked for the agreed contractual schedule duration.'

9. Harmonize Warranty Terminology: Standardize across all sections to '5-Year Written Structural Warranty + 1-Year Comprehensive Maintenance Support'.

10. Add DPDP Act Privacy Consent Microcopy to All Forms: Add consent text under submit buttons: 'By submitting, you agree to receive project consultations via Call/WhatsApp in accordance with our Privacy Policy.'